NETWORK SECURITY AUDIT REPORT

Organization/Target: The Tour Africa – Development Website

Target Domain: dev.thetourafrica.com

Assessment Type: Network & Transport-Layer Security Review

Assessment Date: 11–12 August 2026

Analyst: Tochukwu Oweregbulam

Tools Used: Kali Linux, Wireshark, Nmap, ping, nslookup

1. Executive Summary

A network security assessment was performed against the authorized development website dev.thetourafrica.com.

The assessment focused primarily on:

DNS resolution

Network connectivity

ICMP communication

TCP communication

HTTPS connectivity

TLS encryption

Packet capture and analysis using Wireshark

The target domain successfully resolved to IP addresses including 188.114.96.0 and 188.114.97.0 during the assessment.

ICMP traffic was successfully exchanged between the testing machine and 188.114.96.0. Wireshark also captured TCP traffic to port 443, followed by TLS 1.3 application traffic.

Based on the traffic observed, HTTPS encryption is enabled and TLS 1.3 was observed.

However, this assessment does not by itself establish that the entire network or application is secure. Additional testing of firewall rules, exposed services, TLS certificate configuration, HTTP security headers, authentication, vulnerability management, and network segmentation would be required for a complete security assessment.

2. Scope

The assessment was limited to the authorized development website and network traffic generated by the tester.

Target

Domain: dev.thetourafrica.com

Observed IP: 188.114.96.0

Additional resolved IP: 188.114.97.0

The testing machine was observed using the private address:

172.31.214.192

In Scope

DNS resolution

ICMP connectivity

TCP connectivity

HTTPS/TLS traffic

Packet capture

Basic network-service observation

3. Methodology

The following methodology was used.

Phase 1 — DNS Resolution

The domain was queried using:

nslookup dev.thetourafrica.com

The domain resolved to IP addresses including:

188.114.96.0

188.114.97.0

This confirmed that DNS resolution was functioning.

Phase 2 — Network Connectivity

Connectivity was tested using:

ping dev.thetourafrica.com

ICMP responses were received from:

188.114.96.0

This demonstrated that the target IP was reachable through ICMP from the testing environment.

The observed ping results also showed some packet loss, so network latency/loss should be monitored separately rather than interpreted as a security vulnerability.

4. Wireshark Analysis

Wireshark was used on the eth0 interface to capture network traffic generated during testing.

A display filter was used:

ip.addr == 188.114.96.0

The capture successfully displayed traffic between the testing machine and the target IP.

Observed traffic

The capture showed:

172.31.214.192 → 188.114.96.0

188.114.96.0 → 172.31.214.192

with ICMP traffic observed.

Result

Status: PASS

The target IP was reachable and the resulting packets were successfully captured by Wireshark.

5. HTTPS/TCP Analysis

HTTPS traffic was subsequently observed using TCP port 443.

The following filter was used:

ip.addr == 188.114.96.0 && tcp.port == 443

The capture showed communication involving:

Client temporary port → 443

443 → Client temporary port

The TCP communication included ACK and FIN packets, demonstrating normal TCP communication and connection termination in the captured session.

Result

Status: PASS

HTTPS connectivity to the observed target IP was confirmed.

6. TLS Security Analysis

Wireshark displayed:

TLSv1.3

and TLS application-data packets were observed.

The following filter can be used to isolate TLS traffic:

ip.addr == 188.114.96.0 && tls

Finding

TLS 1.3 was observed

TLS 1.3 provides modern cryptographic protection for HTTPS connections and is generally preferred over older TLS versions.

Result

Status: PASS — Based on observed traffic

However, observing TLS 1.3 alone does not prove that the complete TLS configuration is secure. Certificate validity, certificate chain, cipher suite, key exchange, supported protocols, and configuration of older protocol versions should also be independently verified.

7. Nmap

Nmap was used has the tool to scan the network for any open port

Observation

188.114.96.0 has four open port

Port                Service                  version

80/tcp             http                   cloudflare http proxy

443/tcp            ssl/http            cloudflare http proxy

587/tcp            submission

8080/tcp            http                 cloudflare http proxy

8445/tcp          ssl/http            cloudflare http proxy

With a latency of 0.56s